# Arduino – water dispenser

Description

The pump will dispense water as long as the button is pressed.

Standby status: RED LED ON, GREEN LED OFF

Active pump status: RED LED OFF, GREEN LED ON

# Requirements

| Arduino board |  |
| --- | --- |
| R385 Pump |  |
| 5V Relay |  |
| 9V Battery |  |
| RGB Module |  |
| Other | -cables -hose -button -breadboard |

# Code:

#define RED_PIN 5

#define GREEN_PIN 6

#define BLUE_PIN 7

#define BUTTON_PIN 2

#define RELAY_PIN A0

int buttonStatus = HIGH; // Store the current button status

int pinValue;            // Store the read value from the button

void setup() {

pinMode(RELAY_PIN, OUTPUT);

pinMode(RED_PIN, OUTPUT);

pinMode(GREEN_PIN, OUTPUT);

pinMode(BLUE_PIN, OUTPUT);

pinMode(BUTTON_PIN, INPUT_PULLUP);  // internal pull-up resistor

Serial.begin(9600);

// Safely have everything turned off

digitalWrite(RELAY_PIN, LOW);

digitalWrite(RED_PIN, HIGH);

digitalWrite(GREEN_PIN, LOW);

digitalWrite(BLUE_PIN, LOW);

}

void loop() {

pinValue = digitalRead(BUTTON_PIN);

delay(10); // Debounce delay

if (buttonStatus != pinValue) { // Button state has changed

buttonStatus = pinValue;

if (buttonStatus == LOW) { // Button was pressed

Serial.println("Button pressed!");

digitalWrite(RED_PIN, LOW);

digitalWrite(GREEN_PIN, HIGH);

digitalWrite(RELAY_PIN, HIGH);

} else { // Button was released

Serial.println("Button released!");

digitalWrite(GREEN_PIN, LOW);

digitalWrite(RED_PIN, HIGH);

digitalWrite(RELAY_PIN, LOW);

}

}

}